Apauguma iestrādes tehnoloģijas un alternatīvoorganisko augsnes ielabošanas līdzekļu efektivitātes pārbaude smaga māla akmeņainā augsnē ar nenoturīgu mitruma režīmu audzējot ekoloģiski plastiskus kokaugus Assessment of vegetation removal technologies and the efficiency of alternative organic soil amendment usage for growing resilient woody plants characteristic with high phenotypic plasticity in heavy clay soils

4.1. Vegetation removal and tree harvesting technology trial with and without harvesting the biomass – soil preparation for the amendment trial 4.1. Apauguma, kokaugu atvasāju novākšanas tehnoloģiju izmēģinājums ar un bez biomasas novākšanas – augsnes sagatavošana organiskā augsnes ielabošanas līdzekļa izmēģinājumiem

Autori:

Kristaps Makovskis – apskats,

Dagnija Lazdiņa – teksta tehniskās korekcijas.

Materiāls angļu valodā sagatavots veicot biomasas novākšanas pētījums Skrīveru novada  “Pardenčos”  īstenojot  pētījumu: Inovatīvu Baltā vītola-daudzgadīgo zālaugu agromežsaimniecības sistēmu ierīkošana ar koksnes pelnu un
mazāk pieprasīto kūdras frakciju maisījumiem ielabotās marginālās minerālaugsnēs

Programma "Izaugsme un nodarbinātība" specifiskais atbalsta mērķis 1.1.1. "Palielināt Latvijas zinātnisko institūciju pētniecisko un inovatīvo kapacitāti un spēju piesaistīt ārējo finansējumu, ieguldot cilvēkresursos un infrastruktūrā"
pasākums 1.1.1.1. "Praktiskas ievirzes pētījumi", 3. kārta Nr. 1.1.1.1/19/A/112

Introduction

The results and data were gathered in collaboration with Rural development program project “Cooperation” (16.1) project “Development of harvester prototype for harvesting and biofuel production in short rotation coppice plantations” (agreement No. 19-00-A01620-000089). In he

Methodology

The study was carried in willow plantations, which was established in 2011 in another ERDF project: out as part of the ERDF project: " "Development of establishment and

management models of multifunctional deciduous tree and energy plant plantations" (No. 2010/0268/2DP/2.1.1.2.0/10/APIA/VIAA/118). Work methods were tested in willow fields, where planted tree density was 17 424 cuttings ha-1, willows were planted in double rows with 0.75 m distance, distance between shoots 0.5 m and distance between double row centres is 2.2 meters.

Harvesting productivity was tested in small overgrowth (1-year-old willow plantations), medium overgrowth (3-year-old willow plantations) and heavy overgrowth (5-year-old willow plantations). Plantations were harvested before at least once. Mean single shoot length was 1.5 m (small overgrowth), 3.2 m (medium overgrowth) and 6.3 (heavy overgrowth), with above ground biomass 4.5 t ha-1 (small overgrowth), 16.3 t ha-1 (medium overgrowth) and 40.9 t ha-1 (heavy overgrowth).

The research uses a prototype of a small tree harvester installed on MTZ 82 tractor (Figure 1). The harvester consists of 3 elements - the header, which collect small trees in a bundle, the subframe fixed under the tractor, at the same time covering the less protected parts of the tractor and designing bunch of shoots for cutting and splitting. Making a bundle, using the gravity of the machinery is the main harvester developed advantage, because it allows to significantly reduce the force needed to feed the shoots into the chipper.

Figure 1. Tractor MTZ with harvester device

Work time accounting in the study was carried out with an impact- and moisture-resistant field computer equipped with a specialized program for work time accounting. Work time accounting is done in one working shift (4-6 hours in day) in daylight. Productive working time included drive in the line, moving in the line, harvest, slow harvest (when trees were in dense vegetation), driving out of the field, other works (all other work, what is done with working engine). From working time were excluded chipper cleaning, maintenance, machine adjustments.

Results and conclusions

Removing overgrowth was done in 1 year, 3 year and 5 years old plantations. These plantations ages represent overgrown situations in abandoned agriculture land, where overgrown density and size could vary from small scrubs (1-year-old willow plantations) to very dense and big bushes (5 years old willow plantations) (Figure 2).

Figure 3. Different density overgrowth

Woodchips after harvesting could be collected and sold as biomass, or it can be left on the ground and milled in the soil. In this case study, overgrowth was chipped/shredded and left on the ground for later, when milling should be done (Figure 3).

Figure 3. Overgrowth remove before milling

Each overgrowth scenario small (1 year old willow plantation), medium (3 years old willow plantation) and heavy (5 years old willow plantation) productivity data are summarized in Table 1.

Table 1 Summary of overgrowth productivity in different scenarios

| Indicator | Unit | Small overgrowth (1-year-old plantation) | Medium overgrowth (3-year-old plantation) | Heavy overgrowth (5-year-old plantation) |
| --- | --- | --- | --- | --- |
| Planting density (2011) | piece per ha | 17424 | 17424 | 17424 |
| Cutting survival rate | % | 95% | 80% | 70% |
| Harvesting density | piece per ha | 16553 | 13939 | 12197 |
| Shot from one stem | pieces | 16 | 8 | 4 |
| Harvesting speed | Km per hour | 1,2 | 0,9 | 0,8 |
| Total working time consumption | hours | 4,4 | 5,8 | 6,8 |
| Harvesting productivity | Tons per hour | 1,0 | 2,8 | 6,1 |
|  | m3 per hour | 2.3 | 6.2 | 13.4 |
| Production costs | EUR m-3 | 24,8 | 9,1 | 4,2 |
|  | EUR ton-1 | 55,0 | 20,2 | 9,3 |
|  | EUR loose. m-3 | 10,6 | 3,9 | 1,8 |

The most important factor affecting productivity, as well as the limiting criterion for the application of the technique, is the stock of growing density, diameter and height. The technique is optimally applicable in the plantation of medium overgrowth, although in this case also a more powerful tractor is needed, which can reduce the driving speed while maintaining sufficiently high productivity.

Harvest cost per ton of the equipment is from 55 EUR ton-1 in small overgrowth up to 9.3 EUR ton-1 in heavy overgrowth. However, a realistic indicator of costs is in the medium overgrowth – 20.2 EUR ton-1. In heavy overgrowth, at least at the moment, the capacity of the equipment is insufficient for long-term work in production conditions.

Future improvements

Working in heavy overgrowth requires rebuilding and widening the header that concentrates the stems and the subframe that forms the bundle of stems to be cut, chipped and spread.

Trials have shown that the placement of stems is critical in the sawing process, and the machine is not yet ready for continuous cutting and shredding of natural vegetation at full speed. The main problem is the jamming of the saw when the stumps of the felled trees are bent, if the stumps are cut with the edge of the saw instead of the central part, which is bent closest to the ground. This problem can be solved by replacing circular saws with chain saws. In heavy overgrowth it is necessary to use a more powerful tractor that can drive at a speed of 0.5 km h-1 while ensuring the rotation speed of at least 1000 revolutions per minute and tractor power should exceed 80 HPS.